Data Center Infrastructure Setup – Blade and Fibre Channel SAN Configuration

Blade Infrastructure Overview

The data center environment is built on a Dell PowerEdge M1000e blade chassis, which provides centralized power, networking, and management for multiple blade servers.

Currently, the chassis contains four Dell PowerEdge M610 blade servers. Each blade is equipped with:

- Dual Intel Xeon processors

- 8–16 GB RAM

- Two Fibre Channel Host Bus Adapters (HBAs) for storage redundancy

- Local RAID controller for OS installation

- Ubuntu Server 22.04.5 LTS as the operating system

- The dual HBA configuration allows each blade to connect to two independent Fibre Channel fabrics, ensuring high availability and multipath storage access.

- Inside the M1000e chassis, blades connect through the midplane/backplane architecture to internal switch modules. This eliminates the need for external cabling from each blade.

- The Fibre Channel architecture follows this internal path:

- Blade Server 
   | 
   |  (internal midplane connection) 
   | 
Brocade M5424 FC Switch (C1 or C2) 
   | 
   |  (external OM3 fibre cable) 
   | 
Dell PowerVault MD3600f SAN

- Key concept:

- The HBA in the blade does not communicate directly with the SAN. 
Instead, it first connects to the Fibre Channel switch, which manages communication within the FC fabric.

1. Blade Server Preparation

Before installing the operating system, the blade servers were prepared by configuring their management interfaces and ensuring firmware and drivers were up to date.

iDRAC Configuration

Each blade server includes an Integrated Dell Remote Access Controller (iDRAC) which provides out-of-band management.

The Lifecycle Controller inside the blade server was used to configure the iDRAC network settings. Through this interface the following tasks were performed:

- Assigning a management IP address

- Configuring gateway and subnet settings

- Enabling remote web access to the blade server

- Verifying hardware status and system health

Once configured, the blade servers could be accessed remotely through the iDRAC web interface.

Firmware and Driver Updates

Firmware and driver management for the blade servers was performed through the Dell M1000e Chassis Management Controller (CMC) web interface.

From the CMC interface administrators can:

- View all blades installed in the chassis

- Monitor hardware health

- Update firmware for blade components such as:

- BIOS

- RAID controller

- Network adapters

- Fibre Channel HBAs

- Manage chassis components including power supplies, fans, and switches

Updating the firmware ensures hardware compatibility and prevents issues such as operating system installation failures due to missing RAID controller drivers.

Verification of Blade Hardware

After firmware updates, hardware configuration was verified including:

- RAID controller detection

- Network interface functionality

- Fibre Channel HBA availability

- Memory and CPU configuration

Once these checks were completed, the blades were ready for operating system installation.

Operating System Installation

Ubuntu Server 22.04.5 LTS was installed on the blade servers.

During installation it was observed that:

- Some blades did not initially detect the RAID controller properly.

- This required verifying and updating firmware and drivers.

Once the RAID controller was correctly detected, Ubuntu installation proceeded successfully.

Detailed configuration logs and commands used for the server setup are documented in the project repository:

GitHub Server Logbook 
https://github.com/KMN-Datacenter/Servers/blob/main/GSJM95J/logbook.md

Network Configuration

All blade servers are connected to Switch 131 through a Dell M1000e Ethernet pass-through module.

Network configuration:

- VLAN: 10

- All blades assigned IP addresses in the same management network

- Connectivity verified using SSH and ping tests

This configuration allows the blade servers to be accessed remotely for administration.

| Slot | Hostname | Management | Switch IPv4 | Port | VLAN ID | Hardware | iDRAC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 165 | observium | 172.28.12.1 |  |  |  | 23D3V4J |  |
| 166 | server166 | 172.28.6.166 | 172.28.64.131 | Ethernet1/4 | 2 | BRDM45J | 172.28.15.166 |
| 167 | server167 | 172.28.6.167 | 172.28.64.131 | Ethernet1/5 | 2 | CR70X4J | 172.28.15.167 |
| 168 | server168 | 172.28.6.168 | 172.28.64.131 | Ethernet1/6 | 2 | GSJM95J | 172.28.15.168 |
| 169 | server169 | 172.28.6.169 | 172.28.64.131 | Ethernet1/7 | 2 | 73DQX4J | 172.28.15.169 |

Blade Fibre Channel Identification

Each Fibre Channel device is identified by a WWN (World Wide Name).

This functions similarly to other network identifiers:

| Identifier | Purpose |
| --- | --- |
| MAC Address | Ethernet device identification |
| IP Address | Network communication |
| WWN | Fibre Channel device identification |

To obtain the WWN of the blade HBA ports, the following command was used:

cat /sys/class/fc_host/host*/port_name

Example output:

0x200190b11ceb8eb0 
0x200290b11ceb8eb0

These WWNs uniquely identify the two Fibre Channel HBA ports installed in the blade.

These identifiers are later used during SAN host configuration and LUN mapping.

2. Fibre Channel Infrastructure

To provide centralized storage access, the blade chassis includes two Fibre Channel switches:

- FC Switch C1

- FC Switch C2

These switches form two independent storage fabrics.

Fibre Channel Architecture

Each blade server has two HBA ports connected internally to the chassis midplane:

Blade HBA 1 → Fabric C1 
Blade HBA 2 → Fabric C2

The fabrics are connected to the external storage array.

This architecture ensures:

- Redundant storage paths

- High availability

- Failover capability through multipath storage access

Fibre Channel Connectivity Verification

To verify Fibre Channel connectivity from the blade servers, several Linux commands were used.

Detect Fibre Channel Adapters

lspci | grep -i fibre

Example output:

QLogic ISP2532 8Gb Fibre Channel HBA

This confirms the HBA hardware is detected.

Verify HBA WWN Identifiers

Each Fibre Channel port has a unique World Wide Name (WWN).

Command used:

cat /sys/class/fc_host/host*/port_name

Example output:

0x200190b11ceb8ea3 
0x200290b11ceb8ea3

These identifiers are later used when configuring host access on the SAN.

Verify Fibre Channel Link Status

cat /sys/class/fc_host/host*/port_state

Output:

Online 
Online

This confirms that the Fibre Channel links are active and connected to the switches.

Fibre Channel Switch Management

The Fibre Channel switches inside the blade chassis are accessed remotely from a Windows 7 management VM.

The VM runs Java-based Fibre Channel switch management software, which requires:

Java Runtime Environment

Using this management interface, administrators can:

- View connected HBA ports

- Monitor fabric topology

- Verify SAN connectivity

- Manage zoning configuration

The blade chassis contains two internal Fibre Channel switches:

| Switch | IP Address |
| --- | --- |
| FC Switch C1 | 172.28.10.135 |
| FC Switch C2 | 172.28.10.136 |

These switches are Brocade M5424 Fibre Channel switches installed in fabric slots C1 and C2 of the M1000e chassis.

Access to the switch is performed through SSH:

ssh admin@172.28.10.135

Switch Monitoring Commands

Show Physical Switch Ports

switchshow

Displays:

- Physical FC ports

- Port status

- Connected devices

Show Devices Logged Into Fabric

nsshow

Displays:

- Devices connected to the Fibre Channel fabric

- WWN identifiers

- Fabric port associations

Fibre Channel Terminology

Permanent Port Name

The permanent port name corresponds to the WWN of the blade HBA port.

Example:

20:01:90:b1:1c:eb:8e:96

This identifies the host HBA device.

Fabric Port

The fabric port is the internal switch port on the Fibre Channel switch where the blade connects through the chassis midplane.

Example architecture:

Blade 167 
   | 
   | (midplane connection) 
   | 
Fabric Port (FC Switch C1) 
   | 
   | (external fibre cable) 
   | 
SAN Storage Controller

The FC switch manages device discovery and traffic routing inside the storage fabric.

3. SAN Storage System

The storage system used is a Dell PowerVault MD3600f Fibre Channel SAN.

The SAN contains:

- Dual RAID controllers

- Multiple SAS disks

- Fibre Channel host ports

- Dedicated management Ethernet ports

Each controller connects to the Fibre Channel fabrics using FC switch on M1000e chassis

SAN Network Configuration

Initially, the SAN management interface was configured by connecting a laptop directly to the management Ethernet port.

Steps performed:

- Connect laptop directly to SAN management port.

- Assign a temporary IP address, same as your laptop static ip address

- Access the SAN web interface on latop

- Configure a static management IP address.

After configuration, the SAN was connected to the data center network TOR switch 131 on vlan 10 , allowing remote management.

| SAN1 | IP Address |
| --- | --- |
| SAN1 Controller 1 | 172.28.9.22 |
| SAN1 Controller 2 | 172.28.9.23 |

| SAN2 | IP Address |
| --- | --- |
| SAN2 Controller 1 | 172.28.9.24 |
| SAN2 Controller 2 | 172.28.9.25 |

SAN Management Using MDSM

The SAN is managed using Dell Modular Disk Storage Manager (MDSM) installed on the Windows 7 management VM on PMOX cluster 6 (172.28.2.134).

Using MDSM, administrators can:

- Discover storage arrays

- Configure disk pools

- Create virtual disks (LUNs)

- Map storage to hosts

- Monitor storage health and performance

The storage system used is a Dell PowerVault MD3600f Fibre Channel storage array.

The SAN contains:

- Dual RAID controllers

- 12 SAS physical disks

- Fibre Channel host ports

- Ethernet management ports

Management is performed using Dell Modular Disk Storage Manager (MDSM).

Management VM details:

| VM | IP |
| --- | --- |
| Win7-FC | 172.28.2.150 |

The management software was installed from ISO got from Dell

DELL_MDSS_Consolidated_RDVD_6_5_0_1.iso

Installation type selected:

Full Installation

Event Monitor option:

Manual start

Adding the SAN to MDSM

Inside the MDSM console the SAN was added manually.

Steps:

- Open Dell MD Storage Manager

- Select Add Storage Array

- Choose Out-of-Band Management

- Enter SAN IP address:

172.28.9.22

After discovery the storage array appeared in the management console.

SAN Health Verification

The following hardware components were verified:

| Component | Status |
| --- | --- |
| RAID Controllers | 2 detected |
| SAS Disks | 12 |
| Total Capacity | 21.335 TB |
| Firmware | Verified |
| Operations | None running |

The SAN array reported healthy operational status.

Virtual Machine Migration (Win7-FC) from NOC1 (VMware) to Cluster6 (Proxmox)

The Windows 7 management VM was migrated from VMware Workstation to Proxmox.

Upload VM Files

Using WinSCP, the VMware VM files were uploaded to the Proxmox node:

/var/lib/vz/template/iso/

Create VM in Proxmox

qm create 600 --name "Win7-FC" --memory 4096 --cores 2 --net0 e1000,bridge=vmbr0 --bios seabios --machine pc-i440fx-8.0

Verify VM creation:

qm list

Import VMware Disk

qm importdisk 600 "Windows 7-cl1.vmdk" local-2tb

Attach Disk to VM

qm set 600 --sata0 local-2tb:600/vm-600-disk-0.raw

Configure Boot Disk

qm set 600 --boot order=sata0

Start VM

qm start 600

The Windows 7 management VM is now running on Proxmox and is used for:

- Fibre Channel switch management

- SAN management through MDSM

‘###

#

4. SAN Storage Configuration Procedure

(Disk Pool Creation, LUN Provisioning, Host Registration and LUN Masking)

Fibre Channel Zoning Configuration

After verifying Fibre Channel connectivity between the blade servers, Fibre Channel switches, and the SAN storage array, zoning was configured to control which devices are allowed to communicate within the Fibre Channel fabric.

Zoning is an important security and performance mechanism in Fibre Channel networks. It ensures that only authorized hosts can access specific storage targets.

The configuration process involved the following stages:

- Disk Pool Creation

- Virtual Disk (LUN) Creation

- Host Registration

- LUN Masking (Host Mapping)

Fibre Channel Fabric Layout

The infrastructure uses dual Fibre Channel fabrics for redundancy.

| Fabric | Switch | Purpose |
| --- | --- | --- |
| Fabric A | FC Switch C1 | Primary path |
| Fabric B | FC Switch C2 | Secondary path |

Each blade server has two HBA ports, with each port connected to a different Fibre Channel fabric.

Blade HBA1 → Fabric C1(midplane) → FC Switch1(backplane) → SAN1 Controller1 
Blade HBA2 → Fabric C2(midplane) → FC Switch2(backplane) → SAN1 Controller2

This configuration provides:

- Path redundancy

- Failover capability

- Load balancing through multipath I/O

Identifying Host (Blade) WWNs

The WWN identifiers of the blade HBAs were collected using the following command:

cat /sys/class/fc_host/host*/port_name

Example output:

0x200190b11ceb8eb0 
0x200290b11ceb8eb0

These values represent the permanent port names of the HBA ports and are used during zoning and host registration on the SAN.

Verifying Fibre Channel Fabric Devices

After collecting the host WWNs, the Fibre Channel switches were accessed via SSH to verify device connectivity.

Example access command:

ssh admin@172.28.10.135      ---FC-Switch1

Viewing Switch Port Status

The following command was used to verify active switch ports:

switchshow

This command displays:

- Switch port numbers

- Port status

- Device connection state

Viewing Devices Logged into the Fabric

To identify devices connected to the Fibre Channel fabric, the following command was used:

nsshow

This command lists:

- Host WWN identifiers

- Fabric port numbers

- Connected storage devices

Example interpretation:

| Field | Description |
| --- | --- |
| Permanent Port Name | HBA WWN of the blade server |
| Fabric Port | Internal switch port (midplane) connected to the blade |

Example topology:

Blade Server 
   | 
   | (midplane connection) 
   | 
Fabric Port on FC Switch 
   | 
   | (fibre cable) 
   | 
SAN Storage Controller

This confirms that the blade HBA has successfully logged into the Fibre Channel fabric.

4.1 SAN Disk Pool and LUN Provisioning

After Fibre Channel connectivity and zoning verification, storage resources were configured on the Dell PowerVault MD3600f SAN.

The physical disks in the SAN were grouped into a Disk Pool to provide distributed RAID protection and optimized performance.

Disk Pool Creation

Using Dell Modular Disk Storage Manager (MDSM), a disk pool was created using the available SAS drives.

Configuration:

| Parameter | Value |
| --- | --- |
| Number of disks | 12 |
| Disk type | SAS |
| Total usable capacity | 21.335 TB |

Steps performed in MDSM

- Open Dell Modular Disk Storage Manager (MDSM).

- Select the storage array:

SAN-FC1

- Navigate to:

Storage → Disk Pools

- Select:

Total Unconfigured Capacity

- Click:

Create Disk Pool

- Select available physical disks.

Configuration used:

| Parameter | Value |
| --- | --- |
| Number of disks | 12 |
| Disk type | SAS |
| Disk size | 2 TB |

- Choose disk pool protection level.

Preservation capacity: 2 disks

This reserves capacity for rebuild operations if a disk fails.

- Confirm creation.

Result:

Disk Pool 1 created 
Usable capacity ≈ 21.335 TB

Virtual Disk (LUN) Creation

Logical storage volumes were created from the disk pool.

Each blade server was allocated a dedicated LUN.

Configuration:

| LUN Name | Size | Assigned Host |
| --- | --- | --- |
| LUN166 | 2 TB | Blade Server 1 |
| LUN167 | 2 TB | Blade Server 2 |
| LUN168 | 2 TB | Blade Server 3 |
| LUN169 | 2 TB | Blade Server 4 |

Total allocated storage:

8 TB

Remaining storage:

~13 TB available for future expansion

Steps performed

- In MDSM navigate to:

Storage & Copy Services

- Select:

Create Virtual Disk

- Choose the disk pool:

Disk_Pool_1

- Configure the virtual disk parameters.

Example configuration:

| Parameter | Value |
| --- | --- |
| Virtual disk name | LUN166 |
| Capacity | 2 TB |
| Provisioning type | Thick |
| Cache read prefetch | Enabled |

- Select:

Map later

This allows the LUN to be created before assigning it to hosts.

- Click:

Finish

4.2 Host Registration on SAN

Each blade server was registered as a host in the SAN management interface.

Host configuration example:

| Host Name | Host Type |
| --- | --- |
| blade166 | Linux (DM-MPIO) |

The host was associated with its corresponding HBA WWN identifiers.

Example:

20:01:90:b1:1c:eb:8e:96 
20:02:90:b1:1c:eb:8e:96

These identifiers allow the SAN to recognize which host is requesting storage access.

Host Creation Procedure

- In MDSM open:

Host Mappings

- Right-click:

Unassociated Host Port Identifiers

- Select:

Define Host

- Configure host parameters.

Example:

| Parameter | Value |
| --- | --- |
| Host name | blade166 |
| Host type | Linux (DM-MPIO) |

- Select the WWN identifiers detected by the SAN. (Compare it with whichever blade adding)

Example:

20:01:90:b1:1c:eb:8e:96 
20:02:90:b1:1c:eb:8e:96

- Finish the host creation process.

The blade server is now registered in the SAN.

4.3 LUN Masking (Host Mapping)

After host registration, the storage volumes were mapped to their respective hosts.

This process is called LUN masking.

LUN masking ensures that:

- Only authorized hosts can access specific storage volumes

- Storage devices are isolated between servers

Mapping configuration:

| Host | Assigned LUN | LUN ID |
| --- | --- | --- |
| blade166 | LUN166 | 0 |
| blade167 | LUN167 | 1 |
| blade168 | LUN168 | 2 |
| blade169 | LUN169 | 3 |

This ensures each blade server receives dedicated storage access.

Mapping Procedure

- Navigate to:

Host Mappings

- Right-click:

Undefined Mappings

- Select:

Define Additional Mapping

- Choose the host:

blade166

- Select the virtual disk:

LUN166

- Assign a LUN ID.

Example:

LUN ID = 1

- Confirm the mapping.

Example Mapping Result

| Host | Virtual Disk | LUN ID |
| --- | --- | --- |
| blade166 | LUN166 | 1 |

Each blade server receives its own dedicated LUN.

4.4 Host Storage Discovery

After LUN mapping, the blade servers were instructed to rescan their SCSI devices.

Command used:

echo "- - -" | sudo tee /sys/class/scsi_host/host*/scan

This forces the operating system to detect newly assigned storage devices.

Disk devices can then be verified using:

lsblk

Once detected, the LUN appears as a new block device (e.g., /dev/sdb) which can be formatted and mounted.

4.5 Multipath Storage Configuration   ------------ yet to do

Because each blade server has two Fibre Channel paths to the SAN, multipath I/O is used.

Multipath provides:

- Path redundancy

- Automatic failover

- Improved storage reliability

Multipath status can be verified using:

multipath -ll

This command displays the multiple storage paths available between the host and the SAN.

Current Storage Architecture

The resulting storage architecture is:

Blade Server 
     │ 
     │ HBA Port 1 
     ▼ 
FC Switch C1 
     │ 
     │ Fibre Channel 
     ▼ 
SAN Controller A 
 
Blade Server 
     │ 
     │ HBA Port 2 
     ▼ 
FC Switch C2 
     │ 
     ▼ 
SAN Controller B

Storage resources are delivered through LUNs created on the SAN disk pool and mapped to individual blade servers.

Current Status

The infrastructure now includes:

- Dell M1000e blade chassis

- Four PowerEdge M610 blades

- Dual Fibre Channel fabrics

- Brocade M5424 FC switches

- Dell PowerVault MD3600f SAN

- Disk pool with 21 TB capacity

- Four dedicated LUNs

- Host registration and LUN masking

- Multipath Fibre Channel connectivity   -----------yet to do

| Slot | Hostname | Management | Switch IPv4 | Port | VLAN ID | Hardware | iDRAC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 165 | observium | 172.28.12.1 |  |  |  | 23D3V4J |  |
| 166 | server166 | 172.28.6.166 | 172.28.64.131 | Ethernet1/4 | 2 | BRDM45J | 172.28.15.166 |
| 167 | server167 | 172.28.6.167 | 172.28.64.131 | Ethernet1/5 | 2 | CR70X4J | 172.28.15.167 |
| 168 | server168 | 172.28.6.168 | 172.28.64.131 | Ethernet1/6 | 2 | GSJM95J | 172.28.15.168 |
| 169 | server169 | 172.28.6.169 | 172.28.64.131 | Ethernet1/7 | 2 | 73DQX4J | 172.28.15.169 |

Blades IP below

FC Switch on backplane m1000e – manage via Win7-FC – or can do SSH

| Switch | IP Address |
| --- | --- |
| FC Switch C1 | 172.28.10.135 |
| FC Switch C2 | 172.28.10.136 |

SAN – manage via Win7-FC (MDSM)

| SAN1 | IP Address |
| --- | --- |
| SAN1 Controller 1 | 172.28.9.22 |
| SAN1 Controller 2 | 172.28.9.23 |

| SAN2 | IP Address |
| --- | --- |
| SAN2 Controller 1 | 172.28.9.24 |
| SAN2 Controller 2 | 172.28.9.25 |